Boundary Equipment Checklist

I. Monitoring

| Segment maps | ECBS maps (+ Witness) | Trail map | Compass |
| --- | --- | --- | --- |
| Field notebook | Pencils | Tape measure |  |

II. Maintenance Equipment: Painting

| Paint brushes | Paint bottles | Plastic bags | Scraper or brush |
| --- | --- | --- | --- |

III. Maintenance Equipment: Clearing and Signing

| Loppers | Folding saw | Hammer | Nails |
| --- | --- | --- | --- |
| Washers | Trowel | Signs | Bright flagging tape |

IV. Personal Protective Equipment (PPE)

| Nitrile gloves (pair) | Leather glove (pair) | Safety glasses | Hat or bandana |
| --- | --- | --- | --- |

V. Essentials

| First aid kit | Water | Snacks | Rain gear |
| --- | --- | --- | --- |
| Extra layer | Head lamp | Itinerary - with ETA |  |

I. Monitoring

- Trail map: Use this to get to the boundary.

- Segment maps: Use these to access the boundary and as a reference for type of ownership of the tracts.

- ECBS maps (+ Witness sheet): Essential for boundary bearings, monument locations and witness trees.

- Compass: Learn to use well.

- Field notebook: Notebook, preferably waterproof, for recording field data. When you return from the field, transcribe your field data into the Boundary Monitoring form located on the ATC website.

- Pencils: Bring spares.

- Tape measure: For locating witness trees and monuments.

II. Maintenance Equipment: Painting

- Paint brushes: Use 1 ½ inch wide disposable brushes – the fifty cents kind. They can usually be used for several outings before being thrown away provided that they are kept moist in a plastic bag or with paint thinner. Carry one extra as they sometimes get lost.

- Paint bottles: Expect to use 1-2 quarts of boundary paint in a day of work. Before leaving the trailhead, decant well-mixed gallon containers of paint into quart sized bottles using a funnel. Emptied detergent bottles, drink bottles, shampoo bottles, and condiment bottles all work well for this purpose. When painting a blaze, just pour a dab of paint from the narrow-mouthed bottle onto the brush. Do not store paint in these bottles for more than a week as it will dry out.

- Plastic bags: Carry extras for wet paint brushes, sloppy paint containers, etc.

- Scraper or brush: A scraper can be used to prepare old blazes for repainting. Scrape off only the cobwebs and loose bark.  Do not scrape into the tree. A metal or plastic brush works well for this task too.

III. Maintenance Equipment: Clearing and Signing

- Loppers: long-handled ratcheting loppers work great for clearing most brush and branches from the boundary line. In cases where fast-growing invasive species have taken over the line, mechanical brush cutting tools can be used as well.

- Folding saw: Great tool for cutting large branches or softwood trees that have grown up in the line.

- Claw Hammer: For nailing in signs and witness markers, and for removing old nails.

- Nails: 2-3 inch aluminum nails for sign placement and for witness trees.

- Washers: Aluminum washers for marking witness trees.

- Trowel or rock pick: A metal folding trowel can be used to dig catholes or to find monuments. Rock picks work well for digging for monuments in compacted soils, and can also pound in a nail.

- Signs: Green-on-white 3" x 9" US Boundary signs obtainable from the ATC (for use if you are using yellow paint only) and land management signs such as “yellow meanies,” “No Hunting,” or Forest Service signs

- Bright flagging tape: Flagging tape is used to temporarily mark the boundary before it is cleared and repainted. In cases where surviving blazes are very far apart or separated by large obstacles, it can be left to mark the line. However, flagging is not even semi-permanent and is not a substitute for clearing or painting the line.

IV. Personal Protective Equipment (PPE)

- Nitrile gloves:  To be worn while painting.

- Leather gloves: To be worn while using tools.

- Safety glasses: To be worn at all times on the boundary, particularly while painting. Sunglasses or prescription glasses can be worn instead.

- Hat or bandana: A head covering must be work while painting.

V. Essentials

- First aid kit: Be familiar with all items and how to use them in case of emergency.

- Water: 2-3 liters of water for a day of work.

- Snacks: Always carry more than you think you may need.

- Rain gear: Always keep in pack.

- Extra layer: Such as a fleece or jacket.

- Headlamp: Be prepared to be out after dark.

- Itinerary - with ETA: A cell phone often will not work on the boundary. Leave your itinerary with a friend or family member, along with an estimated time of return. Make a habit of checking in with someone when you leave from and return to the trailhead.